Linux U-Boot中GPIO的控制 v1.0

基于TI AM335x核心平台

免责声明

本文档是作者对GOEMBED 产品进行实际操作和测试后，自我心得总结。建议读者具备一定的计算机基础和基本软件操作能力，如在操作过程中，遇到疑问和错误，欢迎加QQ群(462424566)交流，或发厂商技术支持邮箱进行咨询: support@goembed.com

操作环境配套说明：

| 硬件 | 详细介绍链接 |
| --- | --- |
| SBC3358-B1A单板机 |  |
| 串口调试器：COM10U |  |

| 软件 | 详细介绍链接 |
| --- | --- |
| Ubuntu版本：12.04 LTS（64bit） | http://www.ubuntu.org.cn/download/desktop |
| Linux版本：3.11.0-15-generic |  |
| gcc版本：4.6.3 |  |

SBC3358-B1A单板机软件特性

1、BootLoader版本：u-boot-2013.01.01

2、内核版本：linux-3.2.0

- LCD驱动

- LCD背光驱动

- 电阻式触摸屏驱动

- VGA驱动

- HSMMC/SD/MMC/SDIO驱动

- IIC驱动

- SPI驱动

- 音频驱动

- DMA驱动

- RTC实时时钟驱动

- 电源管理

- USB HOST/DEVICE驱动

- USB OTG驱动

- DEBUG驱动

- 以太网驱动

- TF卡驱动

- CAN驱动

- 串口驱动

- WG驱动

3、交叉工具链：arm-linux-gnueabihf-gcc

SBC3358-B1A单板机资源分配特性

- emmc空间分配

| Partition | Size | Description |
| --- | --- | --- |
| BootLoader | 200MB | FAT32格式分区 |
| rootfs | 约1500MB | EXT3格式分区 |

一、准备工作

1、参考《TI AM335x 搭建Linux开发环境 v1.0.docx》和《TI AM335x Linux系统编译 v1.0.docx》把开发环境搭建好。

2、为了方便阅读和修改代码，在这里我使用的是Source Insight(一个代码编辑工具)对代码进行修改。用户可以直接在终端使用VI编辑器编辑代码，结果是一样的，这里是为了阅读方便。

3、本文以控制一个LED为例子进行说明。

二、修改代码

1、s_init()函数分析

s_init()函数位于u-boot-2013.01.01-psp06.00.00.00\board\ti\am335x\Board.c中，这是一个很重要的函数，完成了时钟、串口等设置，当它调用preloader_console_init()函数之后串口才能打印信息。下面我们在代码中通过添加打印信息来进行调试。s_init()函数如下图所示：

2、配置pinmux

在原理图中可以看到，GPIO1_28连接 的是一个LED灯，我们使用该GPIO：

（1）新增一个struct module_pin_mux结构体变量，该结构体位于u-boot-2013.01.01-psp06.00.00.00\arch\arm\include\asm\arch-am33xx\Mux.h中：

如果要在U-Boot中新增一个GPIO口，则需要在u-boot-2013.01.01-psp06.00.00.00\board\ti\am335x\Mux.c中新增一个对应的结构体：

比如在这里新增GPIO1_28，则新增的结构体如下：

static struct module_pin_mux gpio1_28_pin_mux[] = {

{OFFSET(gpmc_be1n), (MODE(7))},

{-1},

};

需要注意的是，OFFSET(gpmc_be1n)中的gpmc_be1n对应的是该引脚的第一功能，在struct pad_signals 结构体中定义，该结构体位于\u-boot-2013.01.01-psp06.00.00.00\am335x\include\asm\arch\Mux.h中:

如果使用别的引脚，一定要记得把OFFSET(x)中的参数更换为对应的第一功能，比如GPIO3_8则应改为OFFSET（emu1）。那么，怎么知道引脚的第一功能是什么呢？我们可以查看原理图，我们知道很多引脚都有复用功能，在原理图上该引脚的多种功能都会列出，最前面的就是第一功能。

第二个问题，module_pin_mux结构体中的MODE（x）是怎么确定的呢？答案在AM335X.pdf技术手册和数据手册可以看出，每个管脚有8种模式，即MODE0-MODE7，其中MOED0是主模式。在AM335x.pdf中搜索“gpmc_be1n”可以看到，MODE7对应的是gpio1_28模式。所以可以确定MODE为7。其他引脚方法类似。

最后，结构体module_pin_mux最后的“-1”作为结束的判断标记。

（2）、调用void configure_module_pin_mux(struct module_pin_mux *mod_pin_mux);该函数位于u-boot-2013.01.01-psp06.00.00.00\arch\arm\include\asm\arch-am33xx\Mux.h中，我们需要调用它并传入新增的结构体变量gpio1_28_pin_mux对GPIO1_28进行配置。在这里有几个地方需要改，在u-boot-2013.01.01-psp06.00.00.00\board\ti\am335x\Mux.c中新增

void enable_GPIO1_28_pin_mux(void)

{

configure_module_pin_mux(gpio1_28_pin_mux);

}

在u-boot-2013.01.01-psp06.00.00.00\board\ti\am335x\Board.h中新增函数声明，即添加：

void enable_GPIO1_28_pin_mux(void);

然后在u-boot-2013.01.01-psp06.00.00.00\board\ti\am335x\Board.c中的s_init函数调用它，把

enable_GPIO1_28_pin_mux();

添加到最后面即可:

（3）、在s_init中的函数	“enable_board_pin_mux(&header)（位于u-boot-2013.01.01-psp06.00.00.00\board\ti\am335x\Mux.c）”函数中新增：

configure_module_pin_mux(gpio1_28_pin_mux);

（4）、我们需要知道以下几个函数的功能：

- gpio_request(60, " gpio1_28");这个函数的作用是申请GPIO，检查该GPIO口是否可用，如果可用则返回0，不可以则返回-1。这里的“60”是怎么得到的呢？我们知道AM335x有128个GPIO口，也就是说GPIO1_28是第32*1+28=60个，而“gpio1_28”没什么实际用处，可以为空。

- gpio_direction_output(60, 1);将GPIO1_28设置为输出并置为高电平：。

- gpio_set_value（60，0）;将GPIO1_28拉低；

- gpio_set_value（60，1）;将GPIO1_28拉高；

- gpio_direction_input（60）;设置为输入引脚；

- gpio_get_value（60）;获取GPIO1_28的电平。

(5)、加入测试代码：

我们可以在board_late_init()函数中测试该GPIO口，用户也可以在别的初始化函数中使用。这里以board_late_init()中测试为例：

int board_late_init(void)

{

...

//前面的代码不要动

//GPIO1_28 test

int i=0;

if(gpio_request(60,"gpio1_28")==0)

{

gpio_direction_output(60,1);

while(1)

{

gpio_set_value(60,0);

for(i=0;i<1000;i++)

printf("LED_ON\n\r");

gpio_set_value(60,1);

for(i=0;i<1000;i++)

printf("LED_OFF\n\r");

}

}

return 0;

}

三、编译并烧写新的U-Boot

根据《TI AM335x Linux系统编译 v1.0.docx》将U-Boot重新编译并烧写新的U-Boot，开机后不断打印如下信息，并且可以看到核心板上GPIO1_28对应的Led灯一闪一闪（由于原代码在成功进入内核后GPIO1_28控制的LED灯也会一闪一闪，为了避免混淆，测试前请将uImage文件删除，这样系统就不会进入内核）。

U-Boot打印信息如下：

观察核心板可以看到灯一闪一闪，到这里我们成功实现在U-Boot中控制GPIO。

附 相关GOEMBED产品介绍

SBC335x – B1A

SBC335x – B2A

The single board computer SBC335x-B1A/B2A which has an expansion board to carry the CM335X is one of our design of the base plate . The flexible design allows the fast and easy way of realizing and upgrading the controller’s capabilities. In additional to those features offered by CM335X.

The B1A features 4 serial ports (including 2 RS232 and 2 TTL), 4 USB Host and 1 USB OTG, 1 Ethernet ports, CAN, RS485, Wiegand, VGA, LCD, Touch screen, Audio, ADC and more other peripherals.

The B2A features 4 USB Host and 1 USB OTG, 1 Ethernet ports, LCD, Touch screen,RTC, and more other peripherals.

The SBC board targets a wide range of applications, including: HMIs, Digital Signage, POS, Data Terminal, Medical Devices, Navigation, Industrial Automation, Entertainment system, Thin Clients, Robotics, Game Console and much more.

The SBC335x-B1A/B2A are ready-to-run platform to support Linux 3.x, Android 4.x and WinCE 7.0/6.0  operating systems.

If you want to support other Operating System, For more information to contact us.

SBC335x-B1A boards Description of part code:

| Series | B1 | B1 | B1 | B1 |
| --- | --- | --- | --- | --- |
| Part Code | SBC3352 ACW-B1A | SBC3352 BCW-B1A | SBC3358 ACW-B1A | SBC3358 BCW-B1A |
| Order Code | - | - | - | - |
| Core Module | CM3352 ACW -M51E20/08 | CM3352 BCW -M51E40/08 | CM3358 ACW -M51E20/10 | CM3358 BCW -M51E40/10 |
| CPU Type | ARM Cortex™-A8 |  |  |  |
| CPU Cores | 1x |  |  |  |
| CPU Clock | 800MHz | 800MHz | 1.0GHz | 1.0GHz |
| RAM DDR3 | Micron 512MB@16bit*1 |  |  |  |
| eMMC Flash | 2GB@8bit*1 | 4GB@8bit*1 | 2GB@8bit*1 | 4GB@8bit*1 |
| PMU | TI TPS65910A3 |  |  |  |
| Supply Voltage | DC 9-14V |  |  |  |
| Optimal Input | DC 12V,1.5A |  |  |  |
| Size(L*W) | 146 x 102 mm |  |  |  |
| Temperature | 0° to 70° C |  |  |  |
| Support OS | Linux 3.x/ Android 4.x/ Ubuntu/ Angstrom/ Debian/ QT/ WinCE 6.0/7.0 |  |  |  |
| Inventory status | In Stock | Out of Stock Contact us | In Stock | Out of Stock Contact us |
| Minimum Availability | 2022 |  |  |  |

SBC335x-B1A Block Diagram

Figure 1 B1 Block Diagram

SBC335x-B2A boards Description of part code:

| Series | B2A | B2A | B2A | B2A |
| --- | --- | --- | --- | --- |
| Part Code | SBC3352 ACW-B2A | SBC3352 BCW-B2A | SBC3358 ACW-B2A | SBC3358 BCW-B2A |
| Order Code | - | - | - | - |
| Core Module | CM3352 ACW -M51E20/08 | CM3352 BCW -M51E40/08 | CM3358 ACW -M51E20/10 | CM3358 BCW -M51E40/10 |
| CPU Type | ARM Cortex™-A8 |  |  |  |
| CPU Cores | 1x |  |  |  |
| CPU Clock | 800MHz | 800MHz | 1.0GHz | 1.0GHz |
| RAM DDR3 | Micron 512MB@16bit*1 |  |  |  |
| eMMC Flash | 2GB@8bit*1 | 4GB@8bit*1 | 2GB@8bit*1 | 4GB@8bit*1 |
| PMU | TI TPS65910A3 |  |  |  |
| Supply Voltage | DC 9-14V |  |  |  |
| Optimal Input | DC 12V,1.5A |  |  |  |
| Size(L*W) | 130 x 103.5 mm |  |  |  |
| Temperature | 0° to 70° C |  |  |  |
| Support OS | Linux 3.x/ Android 4.x/ Ubuntu/ Angstrom/ Debian/ QT/ WinCE 6.0/7.0 |  |  |  |
| Inventory status | In Stock | Out of Stock Contact us | In Stock | Out of Stock Contact us |
| Minimum Availability | 2022 |  |  |  |

SBC335x-B2A Block Diagram

Figure 1 B2A Block Diagram

ABOUT GOEMBED

GOEMBED team with experienced embedded engineers who have been engaged in ARM hardware and software design for 10+ years.

Our products include single board computers and CPU core modules based on TI ® Sitara and Freescale ® i.MX Applications Processors based on ARM® Cores. Supported by Linux / Android / Debian / Ubuntu / QT / Angstrom / WinCE 7.0 & 6.0 / uCOS. We can redesign carrier boards and SBC as your idea quickly.

GOEMBED focus on Embedded Board Solutions, provide a complete new board for your specified requirement or even a turnkey solution to accelerate your new products to market.

We are your trust worthy partner on ARM embedded design services and solutions.

More Carrier Boards

Customized based on your needs!

ODM / OEM Services

Bring your new products to market quickly

Related end equipment

Learn more applications please click http://www.ti.com/lsds/ti/apps/appshomepage.page